Access Recreation Group, LLC

Cindy Burkhour, MA, CTRS, CPRP

2454 Lamplighter Drive, Jenison, MI  49428-9127

Phone (616)669-9109  Mobile (616)560-2378

AccessRecreationGroup@juno.com

Otter Creek Trail Accessibility Information

Trail Head Location: Esch Beach road end

Date Assessed:  10/24 & 26/2015

| Total Trail Length: 4.5 miles (23,895’) section #1: 2.1 miles (11,217’) section #2: 1.2 miles (6,715’) section #3: 1.1 miles (5,963’) | Overall Barrier Descriptions |
| --- | --- |
| Surface: section #1: packed earth with across trail roots section #2: packed earth with across trail roots section #3: packed earth road bed, ruts, center crown, across trail roots, rocks | Packed earth, cross trail roots, erosion rutting, roadway center crowning, encroaching vegetation |
| Tread Width: section #1: Typical under 36” /Minimum 6” section #2: Typical + 36” /Minimum 13½“ section #3: Typical + 8’ /Minimum 17” | single track narrowed by encroaching vegetation |
| Running Slope: section #1: Typical under 5% /Maximum 17.3% section #2: Typical under 5% / Maximum 13% section #3: Typical under 5% / Maximum 10% | Intermittent steep slope up-hill & down-hill segments |
| Cross Slope: section #1: Typical under 3% / Maximum 11.6% section #2: Typical under 3% / Maximum under 3% section #3: Typical under 3% / Maximum 25% | Road bed crowning/ruts with up to 25% cross slope in straight up/down hill segments with significant soil erosion |

o** Slopes between 5% & 8.33% are equal to the slope on a ramp in the built environment

| Feature Barrier | 603’ | 112’ | 5% | 2 | Information Kiosk & Map Box: Clear space surface is inaccessible grass No accessible route from road |
| --- | --- | --- | --- | --- | --- |
| Feature Barrier |  |  |  |  | Parking: No parking at trail head, roadside only |
| Feature |  |  |  |  | Toilets: None at trail head, nearest at Esch Beach road end, inaccessible design, not signed, not located on an accessible route |
| Feature |  |  |  |  | Potable Water: None |
| Barrier | 609’ | 0’ | 0% | 1 | Surface: inaccessible grass surface Slope: steep slopes 11.4% & 17.3% at roadside ditch on route from road to information kiosk Width: +36” |
| Feature | 163 | 115’ | 2% |  | Ski trail marker |
| Barrier | 164 | 71’ | 7% | 3 | Ski trail marker Surface: packed earth, across trail roots Slope: under 5% Width: +36” |
| Barrier | 165 | 197 | 4% | 4 | Surface: packed earth, 3 mid-trail tree stumps Slope: under 5% Width: +36” |
| Feature | 166 | 140’ | 1% |  | Ski trail marker |
| Barrier Feature | 167 | 81’ | 6% | 5 | Ski trail marker Surface: packed earth Slope: under 5% Width: +36”, then narrows to 30’ |
| Feature | 168 | 42’ | 0% |  | Ski trail marker |
| Feature | 169 | 27’ | 4% |  | Ski trail marker |

| Barrier Feature | 170 | 114’ | 7% | 6 | Ski trail marker Surface: packed earth Slope: under 5% Width: trail narrows further to 21’’ @632’ |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 171 | 92’ | 11% | 7 | Ski trail marker Surface: packed earth Slope: under 5% Width: trail narrows further to 12’’ @731’ |
| Barrier Feature | 172 | 227’ | 0% | 8 | Ski trail marker Surface: packed earth Slope: under 5% Width: trail narrows further to 7”, significant single track rutting |
| Barrier Feature | 173 | 90’ | 0% | 9 | Ski trail marker Surface: packed earth covered with pine needles Slope: under 5% Width: trail widens to 18½” |
| Feature | 174 | 181’ | 1% |  | Ski trail marker |
| Feature | 175 | 78’ | 9% |  | Ski trail marker |
| Barrier Feature | 176 | 77’ | 5% | 10 | Ski trail marker Surface: packed earth, pine needles (slippery when wet) Slope: under 5% Width: narrows to 12” |
| Barrier Feature |  |  |  | 11 | Surface: packed earth, across trail roots Slope: under 5% Width: +36” |

| Barrier Feature | 177 | 116’ | 14% | 12 | Ski trail marker Surface: packed earth Slope: under 5% Width: narrows to 17” |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 178 | 150’ | 3% | 13 | Ski trail marker Surface: packed earth, across trail roots for 6’ Slope: under 5% Width: under 17” |
| Barrier Feature | 179 | 107’ | 3% | 14 15 | Ski trail marker Surface: packed earth, across trail roots Slope: under 5% Width: widens to 24” |
| Barrier Feature | 180 | 111’ | 1% | 16 | Ski trail marker Surface: packed earth Slope: under 5% Width: under 36” |

| Barrier Feature | 181 | 97’ | 1% | 17 18 | Ski trail marker Surface: packed earth, across trail roots (for 8’) Slope: under 5% Width: narrows to 12”, encroaching vegetation |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 182 | 66’ | 0% | 19 20 21 | Ski trail marker Surface: packed earth/muddy, multiple across trail roots Slope: 13.5%, 10.9%, 14.6% Width: continues at 12”, encroaching vegetation |
| Barrier Feature | 183 | 86’ | 0% | 22 | Ski trail marker Surface: packed earth Slope: under 5% Width: narrows to 6”, encroaching vegetation |

| Barrier Feature | 184 | 105’ | 3% | 23 | Ski trail marker Surface: packed earth Slope: under 5% Width: continues at 6”, encroaching vegetation |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 185 | 282 | 1% | 24 25 | Ski trail marker Surface: packed earth/muddy, multiple across trail roots Slope: under 5% Width: widens to 12”, encroaching vegetation |
| Barrier Feature | 186 | 213’ | 0% | 26 | Ski trail marker Surface: packed earth/muddy, multiple across trail roots Slope: under 5% Width: continues at 12”, encroaching vegetation |
| Barrier Feature | 187 | 123’ | 1% | 27 28 | Ski trail marker Surface: packed earth, across trail roots Slope: under 5% Width: continues at 12”, encroaching vegetation |

| Barrier Feature | 188 | 328’ | 2% | 29 | Ski trail marker Surface: packed earth, across trail roots + 3½” Slope: under 5% Width: continues at 12”, encroaching vegetation |
| --- | --- | --- | --- | --- | --- |
| Feature | 189 | 234’ | 0% |  | Ski trail marker |
| Barrier Feature | 190 | 343’ | 2% | 30 | Ski trail marker Surface: packed earth Slope: under 5% Width: continues at 12”, encroaching vegetation |
| Barrier | 191 | 169’ | 1% | 31 32 | Ski trail marker Surface: packed earth, cross trail roots (cedar trees) Slope: under 5% Width: widens to 18” |
| Barrier Feature | 192 | 116’ | 2% | 33 | Ski trail marker Surface: packed earth, cross trail roots (cedar trees) Slope: under 5% Width: continues at 18” |

| Barrier Feature | 193 | 528’ | 4% | 34 35 | Surface: packed earth, changing to crush/fines @ 4000’, very large & many cross trail roots (cedar trees), crowned surface for 42’ Slope: up to 7% - 11.2% Width: begins at 18” then widens to 72” |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 194 | 248’ | 0% | 36 | Ski trail marker Surface: packed earth, cross trail roots Slope: up to 11.2% (for 24’) Width: @ 4593’ narrows dramatically, double crowning ends @ 4694 |
| Barrier Feature | 195 | 1056’ | 0% | 37 38 | Ski trail marker Surface: packed earth, multiple crowns for 36’, multiple large across trail roots Slope: under 5% Width: widens to 21” |

| Barrier Feature | 196 | 1056’ | 0% | 39 40 | Ski trail marker Surface: packed earth, multiple large across trail roots Slope: under 5% Width: narrows to 10”, then 7” |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 197 | 1056’ | 0% | 41 42 | Ski trail marker Surface: packed earth, multiple large across trail roots +3” Slope: under 5% Width: widens to 18”, host to one very cool snake (see photo!) |
| Barrier Feature | 198 | 170’ | 15% | 43 | Ski trail marker Surface: sandy soil, very large ferns Slope: under 5% Width: narrow single track 6”, encroaching vegetation |
| Barrier Feature | 199 | 204’ | 15% |  | Ski trail marker Surface: packed earth Slope: under 5% Width: remains narrow 6” |
| Barrier Feature | 200 | 216’ | 0% | 44 | Ski trail marker Surface: packed earth, cross trail roots Slope: under 5% Width: +36” |

| Barrier Feature | 201 | 130’ | 1% | 45 | Ski trail marker Surface: packed earth, some loose/soft sand Slope: under 5% Width: narrows to 18” |
| --- | --- | --- | --- | --- | --- |
| Barrier |  |  |  | 46 47 48 | Surface: packed earth multiple large across trail roots +2” - +4” Slope: up to 14.6% with cross slopes up to 11.6% on approach to bridge Width: +36” |

| Barrier Feature |  |  |  | 49 50 51 | Surface: boardwalk bridge, 6” change of level onto bridge Slope: under 5% Width: 45” w/16” drop-off w/out edge protection at ramped exit from the bridge |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 202 | 205’ | 0% |  | Ski trail marker Surface: packed earth Slope: 12.3%, 8.7%, 7.5% Width: +36” |
| Barrier Feature | 203 | 172’ | 4% |  | Ski trail marker Surface: packed earth Slope: under 5% Width: narrows to 6”, encroaching vegetation |

| Barrier Feature | 204 | 1056’ | 0% | 52 53 54 55 | Ski trail marker Surface: packed earth, multiple across trail roots +1½”, +2½”, +3½” Slope: under 5% Width: widens to +36”, social trail inter section |
| --- | --- | --- | --- | --- | --- |

| Barrier Feature |  |  |  | 56 57 58 | Surface: packed earth, mossy crowning, multiple across trail roots 2½”, +3½” Slope: under 5% Width: +36” |
| --- | --- | --- | --- | --- | --- |
| Section #2 |  |  |  |  |  |
| Barrier Feature | 205 | 1214’ | 0% | 61 | Surface: packed earth Slope: 11.4% Width: 13½” |
| Barrier | 206 | 57’ | 25% | 62 63 | Surface: packed earth, crowned road bed Slope: under 5% Width: +60” |

| Barrier Feature | 207 | 154’ | 6% | 64 | Ski trail marker Surface: packed earth Slope: under 5% Width: +60” |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 208 | 897’ | 0% | 65 66 67 | Trail sign Surface: packed earth, mid-trail stumps at 708’ & 972’ Slope: under 5% Width: +60” narrows to 40” at 1117’ |

| Barrier Feature | 209 | 792’ | 0% | 68 69 70 71 | Ski trail marker Surface: packed earth & gravel road bed, Slope: under 5% Width: +9½’ Cabin at 1479’; boat launch carry down site & lake access at 1941’ |
| --- | --- | --- | --- | --- | --- |

| Barrier Feature | 210 | 1050’ | 0% | 72 73 74 75 76 77 | Trail intersection to trail gate Surface: packed earth & gravel road bed with washouts Slope: under 5% Width: +9½’, narrows to 17” between gate post & tree |
| --- | --- | --- | --- | --- | --- |
| Barrier Feature | 211 | 320’ | 3% |  | Ski trail marker Surface: packed earth & crowned gravel road Slope: under 5% Width: +7½‘ |

| Barrier Feature | 212 | 236’ | 0% | 78 | Ski trail marker Surface: changes to packed earth, crush/fines begin at 3193’ & ends at 3400’. Slope: 8%, 6.6%, 11.9% Width: narrows to 36” |
| --- | --- | --- | --- | --- | --- |
| Barrier |  |  |  | 79 | Surface: packed earth, rolling terrain, across trail roots Slope: 11.9%, 9.1%, 7.2%, 11%, 7.2 Width: +36” |
| Barrier |  |  |  | 80 | Surface: packed earth, rolling terrain Slope: 9.1%, 13%, large dip at 3906’ Width: +36” |
| Barrier Feature | 213 | 1320’ | 0% | 81 82 83 | Surface: packed earth, across trail roots +4½“ & 3½“ Slope: under 5% Width: +36”, narrows to 21” at 4683’ Social trail to Otter Lake, route reduced to 25” through downed tree cut-out |
| Feature | 214 | 528’ | 0% |  | Ski trail marker |

| Barrier Feature | 215 | 1478’ | 0% | 84 85 86 87 88 89 90 | Surface: packed earth, multiple segments of across trail roots +2” Slope: under 5% Width: continues at 21” to trail end at gate |
| --- | --- | --- | --- | --- | --- |
| Section #3 |  |  |  |  |  |
| Barrier |  |  |  | 1 | Surface: packed earth road bed, ruts, center crown Slope: under 5% Width: +8’ |
| Barrier Feature | 216 | 1584’ | 0% | 2 3 | Ski trail marker Surface: packed earth road bed, ruts, center crown, across trail roots Slope: under 5% w/cross slopes up to 25.5% due to crowning/ruts Width: +8’ |
| Barrier | 217 | 4224’ | 0% | 4 5 | Surface: packed earth road bed, gently rolling hills, a few across trail roots & some crowning Slope: under 5% Width: +8’ |

| Barrier |  |  |  | 6 | Surface: packed earth road bed, ruts, center crown, across trail roots, rocks Slope: 10%, 6.5%, 5%, 9.5%, 7% Width: +8’ |
| --- | --- | --- | --- | --- | --- |
| Barrier |  |  |  | 7 8 9 | Surface: packed earth road bed, ruts, center crown, across trail roots, rocks Slope: 8%, 7.5% Width: +8’ |

| Barrier | 218 | 301’ | 0% | 10 11 12 | Surface: packed earth road bed, ruts, center crown, across trail roots, rocks Slope: under 5% Width: +8’ |
| --- | --- | --- | --- | --- | --- |
| Barrier |  |  |  | 13 14 | Surface: packed earth road bed, deep ruts, ponding water, center crown, across trail roots, rocks Slope: under 5% Width: +8’ |

| Barrier |  |  |  | 15 16 | Surface: wood surfaced bridge, 4” change of level onto bridge Slope: under 5% Width: +8’ |
| --- | --- | --- | --- | --- | --- |
| Barrier |  |  |  | 17 | Surface: wood surfaced bridge, 3” change of level off bridge Slope: under 5% Width: +8’ |